ICT Framework

Developer Documentation

SPEA_3030 Demo — Revision 1.0

# 1. Overview

This framework is a demonstration ICT (In-Circuit Test) automation system built around the SPEA 3030 tester. It provides a thin but complete vertical slice from hardware communication up to an operator-facing GUI, covering the following responsibilities:

- Loading a test plan from an XML configuration file

- Dispatching named tests to the tester controller

- Enforcing a critical-test abort policy

- Logging pass/fail results to a rotating log file

- Presenting a Tkinter-based operator GUI with live status feedback

The framework is intentionally simple: it simulates hardware responses with time-delays rather than making real SPEA API calls, which makes it runnable on any development machine without lab equipment.

# 2. Project Structure

The repository is organised into three Python packages and two top-level files:

ict_framework/

controllers/

base_controller.py      # Abstract base class (ICTControllerBase)

spea_3030.py            # Concrete SPEA 3030 implementation

core/

test_sequence.py        # Orchestrates test execution

result_logger.py        # Wraps Python logging

barcode_reader.py       # Serial-number acquisition

fixture_control.py      # Thin fixture open/close wrappers

ui/

operator_gui.py         # Tkinter operator interface

main_gui.py               # Application entry point

testplan.xml              # Declarative test plan

Design principle: each layer depends only on the layer below it. The GUI talks to TestSequence, which talks to a controller that satisfies ICTControllerBase. Swapping the SPEA 3030 for a different tester requires only a new controller class — no changes to core or UI.

# 3. Test Plan — testplan.xml

The test plan is a simple XML file that lists the tests to execute and flags which ones are critical:

<testplan>

<test name="POWER_ON"      critical="true" />

<test name="SHORTS"        critical="true" />

<test name="OPENS"                          />

<test name="ANALOG_MEAS"                   />

<test name="BOUNDARY_SCAN"                 />

</testplan>

Each <test> element carries two meaningful attributes:

| Attribute | Meaning |
| --- | --- |
| name | Unique test identifier passed verbatim to the controller's run_test() method |
| critical | Optional boolean (default false). When true, a FAIL on this test aborts all remaining tests immediately |

The file is parsed in two places: once inside SPEA3030Controller.load_testplan() (which stores the XML tree) and once inside load_tests_from_xml() in main_gui.py (which produces the plain list of dicts consumed by TestSequence).

# 4. Controllers Layer

## 4.1 ICTControllerBase — Abstract Interface

Located in controllers/base_controller.py. This is an abstract base class (ABC) that defines the complete contract any tester controller must fulfil:

class ICTControllerBase(ABC):

@abstractmethod

def connect(self): ...

@abstractmethod

def load_testplan(self, path: str): ...

@abstractmethod

def list_tests(self): ...

@abstractmethod

def run_test(self, test_name: str): ...

@abstractmethod

def fixture_close(self): ...

@abstractmethod

def fixture_open(self): ...

@abstractmethod

def disconnect(self): ...

Using an ABC here enforces the interface at class definition time. Python raises a TypeError if a subclass is instantiated without implementing every abstract method, which catches integration errors early.

The interface also enables dependency-inversion in the rest of the code: TestSequence and fixture_control.py both accept any ICTControllerBase, making unit-testing and tester substitution straightforward.

## 4.2 SPEA3030Controller — Concrete Implementation

Located in controllers/spea_3030.py. Implements all seven abstract methods for the SPEA 3030 tester. Because this is a demo, hardware calls are replaced by time.sleep() delays and print statements.

| Method | Behaviour (demo) |
| --- | --- |
| connect() | Prints a connection message, sleeps 0.5 s, sets self.connected = True |
| load_testplan(path) | Parses testplan.xml into self.tree / self.root via xml.etree.ElementTree |
| list_tests() | Returns a list of test name strings extracted from self.root |
| run_test(test_name) | Sleeps 0.3 s then returns a hard-coded PASS dict {name, status, details} |
| fixture_close() | Prints a message and sleeps 0.5 s |
| fixture_open() | Prints a message and sleeps 0.5 s |
| disconnect() | Prints a message and sets self.connected = False |

In production, each method would wrap the real SPEA API — typically a TCP socket, a Windows DLL, or a command-line interface. Only SPEA3030Controller needs to change; no other file is touched.

# 5. Core Layer

## 5.1 TestSequence

Located in core/test_sequence.py. The heart of the framework. Its single public method run_sequence() accepts a serial number and a list of test descriptors and drives the entire test run:

def run_sequence(self, serial: str, tests: List[Dict]) -> list[dict]:

results = []

self.logger.log_info(f'=== Starting ICT for SN={serial} ===')

for t in tests:

name     = t['name']

critical = t.get('critical', False)

result = self.controller.run_test(name)   # delegate to controller

results.append(result)

if result['status'] != 'PASS':

self.logger.log_failure(name, result.get('details', ''))

if critical:

self.logger.log_info('Critical test failed. Aborting.')

break                              # early exit

self.logger.log_summary(serial, results)

return results

Key behaviours to note:

- Critical abort: if a test marked critical returns anything other than 'PASS', the loop breaks immediately. Tests after the failing test are never executed and do not appear in the results list.

- Non-critical failures: execution continues past a non-critical failure; all remaining tests are still run.

- Return value: a list of result dicts (one per executed test) is returned to the caller, which in practice is the GUI. The GUI uses this list to set the final PASS/FAIL banner.

## 5.2 ResultLogger

Located in core/result_logger.py. A thin wrapper around Python's standard logging module that writes timestamped records to ict_results.log while also printing to stdout.

| Method | What it logs |
| --- | --- |
| log_info(msg) | INFO level — general progress messages |
| log_failure(test_name, details) | ERROR level — 'FAIL: <name> - <details>' |
| log_summary(serial, results) | INFO level — overall PASS/FAIL for a board |

The logger is constructed once in main_gui.py and injected into TestSequence. Because both the GUI and the sequence share the same logger instance, all messages flow to the same log file.

## 5.3 BarcodeReader

Located in core/barcode_reader.py. In this demo, read_barcode() calls input() to prompt the operator at the terminal. In production, this would query a USB HID scanner or a serial port and return the decoded string.

In the GUI flow, the serial number is instead typed directly into the Tkinter entry field, so this module is available as a building block but is not wired up to the GUI path.

## 5.4 FixtureControl

Located in core/fixture_control.py. Two one-line convenience functions:

def close_fixture(controller: ICTControllerBase):

controller.fixture_close()

def open_fixture(controller: ICTControllerBase):

controller.fixture_open()

These wrappers give the rest of the codebase a named action rather than a direct method call, which keeps the core layer's vocabulary consistent and makes it easy to add pre/post logic (interlocks, sensor checks) without touching the GUI or controller.

# 6. UI Layer — OperatorGUI

Located in ui/operator_gui.py. A Tkinter window (600 × 500 px) that provides a minimal operator interface. It is constructed with four dependencies passed in at creation time: root (the Tk instance), controller, test_sequence, and load_tests_func (the XML loader). On construction the GUI also creates an internal queue.Queue and starts the polling loop that keeps all widget updates on the main thread.

## 6.1 Widget Layout

| Widget | Purpose |
| --- | --- |
| serial_entry (ttk.Entry) | Operator types or scans the board serial number |
| start_button (ttk.Button) | Triggers start_test_thread(); disabled for the duration of a run to prevent concurrent submissions |
| results_box (tk.Text) | Scrollable read-only log of test progress during the run |
| status_label (ttk.Label) | Large PASS (green) or FAIL (red) banner shown at run end |

## 6.2 Threading Model

Clicking Start calls start_test_thread(), which runs on the main thread, disables the Start button to block re-entry, then spawns a daemon Thread targeting _run_test(). Worker threads are necessary because the SPEA calls (even the simulated time.sleep() ones) would freeze the Tkinter event loop if run on the main thread.

Tkinter is not thread-safe: widget methods must only be called from the main thread. The GUI enforces this through a queue.Queue and a polling loop:

- Worker thread: never calls any widget method directly. Instead it puts plain Python dicts onto self._queue — for example {"kind": "log", "text": "..."} or {"kind": "status", "passed": True}. A queue.Queue is thread-safe by design, so this operation carries no risk.

- Main thread — _poll(): registered with root.after(50, self._poll) at construction time and rescheduled at the end of every call. Every 50 ms it drains the queue with get_nowait() in a tight loop until queue.Empty is raised, then dispatches each message to _handle(). Because _poll and Tkinter's own event callbacks share the same thread, widget updates inside _handle are always single-threaded and race-free.

- Message types: the _handle() method dispatches on the kind field: "log" appends a line to the results box; "status" sets the PASS/FAIL banner; "button" enables or disables the Start button; "clear" resets the results box and banner at the start of a new run.

The Start button is disabled on the main thread before the worker is spawned, and re-enabled via a {"kind": "button", "state": "normal"} message at the end of the run. This prevents the operator from starting a second run while one is already in progress.

## 6.3 Test Run Sequence (GUI)

The worker function _run_test(serial) performs the following steps, communicating with the GUI exclusively through the queue:

- Put a "clear" message — results box is wiped and banner is blanked

- Put a "log" message announcing the serial number

- Call controller.fixture_close() — board is clamped

- Call load_tests('testplan.xml') — re-reads the XML every run, so plan edits take effect without restart

- Call test_sequence.run_sequence(serial, tests) — blocks until complete; TestSequence calls back into self.log() which puts messages onto the queue

- Call controller.fixture_open() — board is released

- Put a "status" message with the overall pass/fail result

- Put a "button" message to re-enable the Start button

# 7. Application Entry Point — main_gui.py

The main() function is the only place in the codebase where all components are wired together:

def main():

controller = SPEA3030Controller('testplan.xml')

logger     = ResultLogger()

seq        = TestSequence(controller, logger)

controller.connect()

controller.load_testplan('testplan.xml')

root = tk.Tk()

gui  = OperatorGUI(root, controller, seq, load_tests_from_xml)

root.mainloop()          # blocks until window is closed

controller.disconnect()

A few details worth noting:

- load_testplan() is called once at startup, but the GUI also calls load_tests_from_xml() on every test run. The startup call pre-populates the XML tree in the controller (e.g. for list_tests()); the per-run call produces the typed list consumed by TestSequence.

- controller.disconnect() executes after root.mainloop() returns, i.e., after the operator closes the window. This guarantees a clean shutdown regardless of how the window was closed.

- The load_tests_from_xml function defined in main_gui.py is passed to the GUI as a callback rather than imported directly by the GUI. This keeps OperatorGUI free of any dependency on the XML parsing logic.

# 8. End-to-End Data Flow

The following trace follows a single board through the system:

1.  Operator enters serial number in GUI entry field.

2.  Operator clicks Start Test  ->  main thread disables button,

spawns daemon worker thread.

3.  Main thread continues running _poll() every 50 ms.

4.  Worker puts {kind:'clear'} on queue.

_poll drains it  ->  results box wiped, banner blanked.

5.  Worker calls controller.fixture_close()  ->  board clamped.

Worker puts {kind:'log', text:'Closing fixture...'} on queue.

6.  Worker calls load_tests_from_xml('testplan.xml')

->  returns [{name:'POWER_ON', critical:True}, ...]

7.  Worker calls seq.run_sequence(serial, tests)

For each test:

a. logger.log_info(...)  ->  worker puts {kind:'log'} on queue

b. controller.run_test(name)  ->  {name, status:'PASS', details:''}

c. If FAIL and critical  ->  break loop

d. logger.log_failure(...)  ->  puts {kind:'log'} on queue

logger.log_summary(serial, results)

Returns results list.

8.  Worker calls controller.fixture_open()  ->  board released.

9.  Worker puts {kind:'status', passed:True/False} on queue.

Worker puts {kind:'button', state:'normal'} on queue.

_poll drains both  ->  banner updated, button re-enabled.

All logging calls in step 7 write simultaneously to stdout and to ict_results.log. The queue drains asynchronously during the run — each _poll cycle (every 50 ms) picks up whatever messages the worker has posted since the last cycle.

# 9. Extension Points

The framework is designed to be extended at well-defined seams:

| What to extend | How |
| --- | --- |
| New tester hardware | Subclass ICTControllerBase and implement the seven abstract methods. Pass the new controller to TestSequence and OperatorGUI — nothing else changes. |
| Different test plan format | Replace load_tests_from_xml() with a function that returns the same list-of-dicts schema. Pass the new function as load_tests_func to OperatorGUI. |
| Real barcode scanner | Replace the input() call in barcode_reader.read_barcode() with the appropriate HID or serial read, then wire the return value into the serial_entry field on startup. |
| New GUI message types | Add a new 'kind' string to the dict protocol and a matching branch in _handle(). The worker puts messages; _handle() acts on them — no other file needs to change. |
| Richer logging | Add a log_pass() method to ResultLogger, or switch from rotating files to a database by changing the logging back-end inside that class. |

# 10. Dependencies

The framework uses only the Python standard library. There are no third-party packages to install:

| Module | Used for |
| --- | --- |
| tkinter / tkinter.ttk | Operator GUI (included with CPython on all platforms) |
| queue.Queue | Thread-safe message channel between worker and main/GUI thread |
| xml.etree.ElementTree | XML testplan parsing |
| logging | Timestamped file and console logging |
| threading.Thread | Non-blocking test execution in GUI |
| pathlib.Path | Cross-platform file path handling |
| time.sleep | Hardware delay simulation (demo only) |
| abc.ABC / abstractmethod | Abstract base class enforcement |

Python version: The code uses list[dict] return-type hints (lowercase list, introduced in Python 3.9) so Python 3.9 or later is required.

End of Document